Basic Firewall Rules (iptables)

- Allows all packets belonging to an existing connection

iptables -A INPUT -m state --state RELATED,ESTABLISHED -j ACCEPT

- Secure SSH Access (Port 22)

- allows SSH access (port 22) ONLY from the specific source IP address 10.253.101.34 (white listed ip address)

iptables -A INPUT -s 10.253.101.34/32 -p tcp -m tcp --dport 22 -j ACCEPT

- Logs all NEW connection attempts with SYN flag to port 22 ( for finding the brute force attempt)

iptables -A INPUT -p tcp -m tcp --dport 22 --tcp-flags FIN,SYN,RST,ACK SYN -j LOG --log-prefix "SSH-BLOCKED: "

- Drops all remaining traffic destined for port 22.

Iptables -A INPUT -p tcp -m tcp --dport 22 -j DROP

3) SYN Scan/Flood Protection

- Marks the source IP  using (--rsource) of any packet with the SYN flag set into synscan

Iptables -A INPUT -p tcp -m tcp --tcp-flags FIN,SYN,RST,ACK SYN -m recent --set --name synscan --mask 255.255.255.255 --rsource

- This rules log the event  with the source IP attempts 8 or more new connections with in one second

Iptables -A INPUT -p tcp -m tcp --dport 22 --tcp-flags FIN,SYN,RST,ACK SYN -j LOG --log-prefix "SSH-BLOCKED: "

4)    Single  SYN  protection ( example with FTP)

- Logs any attempt to FTP (a single SYN packet)

Iptables  -A INPUT -p tcp -m tcp --dport 21 --tcp-flags FIN,SYN,RST,ACK SYN -m state  --state  NEW -j LOG --log-prefix "FTP-SINGLE-SYN-SCAN: "

- Drops all traffic to port 21

Iptables -A INPUT -p tcp -m tcp --dport 21 -j DROP

5)  FIN Scan Detection

- Logs packets where only the FIN flag is set

Iptables -A INPUT -p tcp -m tcp --tcp-flags FIN,SYN,RST,PSH,ACK,URG FIN -m limit --limit 5/min -j LOG --log-prefix "NMAP_FIN_SCAN: "

- Drops all packets that have only the FIN flag set

Iptables -A INPUT -p tcp -m tcp --tcp-flags FIN,SYN,RST,PSH,ACK,URG FIN -j DROP